NEURO-SHIELD

Neural Emergency & Road Optimization Shield for BIMSTEC

Road Safety Hackathon 2026

Software Packages, Assumptions & Database Documentation

# 1.  Project Overview

NEURO-SHIELD is a 100% Python road safety system built for the BIMSTEC Road Safety Hackathon 2026. It detects crashes using an AI model on an edge device, alerts first responders in real time, and predicts dangerous road zones before they become official blackspots.

The entire system — web server, dashboard, AI models, and data pipeline — runs with a single command: python app/main.py. No Node.js, no React, no external build tools.

The system has three layers:

- Sentinel Node (Edge): ESP32 + IMU sensor running LSTM TFLite crash classifier

- Responder Mesh (Flask API): V2X broadcast, QR Medical ID, SSE real-time push, TTS

- Grey-Spot Predictor (ML): K-Means clustering + Random Forest blackspot prediction

# 2.  How to Run

## Windows (one command)

.\run.bat

This creates a virtual environment, installs all dependencies, and starts the server at http://localhost:8000

## Simulate a Crash

.\run_emulator.bat  →  type  c  →  Enter

Crash appears live on the Hospital tab with map marker, alert sound, and rider medical data.

## Train AI Models

.\train_models.bat

Trains the LSTM crash classifier and Random Forest grey-spot predictor. Pre-trained models are included in the submission.

# 3.  Software Packages

## 3.1  Backend — Python (Flask)

| Category | Package | Version | Purpose |
| --- | --- | --- | --- |
| Web Framework | flask | 3.1.0 | HTTP server, routing, Jinja2 templates, SSE streaming |
| Templates | Jinja2 | 3.1.6 | Server-side HTML rendering — all 5 dashboard pages |
| QR Codes | qrcode[pil] | 7.4.2 | Generates QR PNG images for Medical ID helmet stickers |
| Image | Pillow | 10.3.0 | Required by qrcode for PNG rendering |
| Numerics | numpy | 1.26.4 | IMU signal processing, sliding window creation for LSTM |
| HTTP Client | requests | 2.31.0 | Used by edge emulator to POST crash events to API |
| Config | python-dotenv | 1.0.1 | Loads .env variables at startup |
| Real-time | Built-in SSE | — | Server-Sent Events via Flask Response + threading.Queue — no extra library |

## 3.2  Machine Learning — Python

| Category | Package | Version | Purpose |
| --- | --- | --- | --- |
| ML Framework | scikit-learn | 1.4.2 | K-Means clustering + Random Forest classifier for grey-spot pipeline |
| Deep Learning | tensorflow | 2.16.1 | Optional. LSTM model training. Falls back to RandomForest if not installed |
| Edge AI | TFLite Micro | built-in | Converts LSTM to .tflite for ESP32 deployment. Under 50KB, under 2ms inference |
| Model Save | joblib | 1.4.2 | Saves and loads trained ML models to .joblib files |
| Geospatial | geopandas | 0.14.4 | Spatial binning and GeoJSON export for grey-spot heatmaps |
| Data | pandas | 2.2.2 | Near-miss event tabulation and feature engineering |
| Numerics | numpy | 1.26.4 | Vectorized IMU time-series operations |

## 3.3  Frontend — Served by Flask (no npm needed)

| Category | Package | Version | Purpose |
| --- | --- | --- | --- |
| Map Library | Leaflet.js | 1.9.4 | Interactive maps for crash location and grey-spot heatmap. Loaded via CDN |
| Tile Provider | CartoDB Dark | CDN | Dark OpenStreetMap tiles for both maps |
| SSE Client | EventSource API | browser built-in | Receives real-time crash events from Flask SSE endpoint. No library needed |
| Audio | Web Audio API | browser built-in | 3-tone alert sound on crash arrival. No library needed |

## 3.4  Edge / IoT Hardware

| Category | Component | Type | Purpose |
| --- | --- | --- | --- |
| Microcontroller | ESP32-S3 | Hardware | Main compute unit. WiFi + BLE 5.0 built-in. Runs MicroPython + TFLite |
| IMU Sensor | MPU-6050 | Hardware | 6-axis accelerometer + gyroscope. I2C. 100Hz sampling rate |
| GPS | NEO-6M | Hardware | UART GPS for crash location tagging |
| LoRa Radio | SX1276 | Hardware | LoRaWAN 5–15km range. Offline fallback for rural BIMSTEC areas |
| Firmware | MicroPython 1.22 | Software | Reads IMU, runs TFLite inference, sends alerts |

# 4.  Assumptions

| Area | Assumption |
| --- | --- |
| Hardware | ESP32/IMU/GPS/LoRa hardware assumed available for production. Submission uses edge_emulator.py which generates equivalent synthetic IMU data streams. |
| ML Training | LSTM trained on synthetic IMU data. In production, real crash recordings from instrumented vehicles replace synthetic data. 100% accuracy shown is on the synthetic test set. |
| ML Fallback | If TensorFlow not installed, train_lstm.py uses RandomForest from scikit-learn. The edge emulator uses a rule-based classifier that mirrors the LSTM logic exactly. |
| Database | All data stored in Python in-memory lists and dicts. No database setup required. Data resets when server restarts. Production would use PostgreSQL + PostGIS. |
| SSE | Server-Sent Events implemented with threading.Queue per subscriber. Flask runs with threaded=True. Production would use Redis pub/sub for higher concurrency. |
| V2X Broadcast | V2X simulated via HTTP API calls. Production uses Bluetooth LE advertising and WiFi Direct for under 200ms latency to nearby physical vehicles. |
| LoRaWAN | Offline mesh protocol designed and coded into services.py. Physical LoRa hardware not included in hackathon submission. |
| Grey-Spot Data | Bengaluru and Dhaka grey-spot coordinates use real road locations. Near-miss counts are simulated. Production data comes from live Sentinel Node deployments. |
| Federated Learning | Privacy boundary implemented — no raw GPS leaves the device. Full federated server (PySyft/Flower) not included — required for multi-country production. |
| TTS Audio | API returns translated message string. Audio synthesis needs Google TTS or Azure Speech in production. Browser handles audio playback via Web Audio API. |
| Medical ID | Stored in Python dict. Production uses AES-256 encrypted database with role-based access limited to verified first responders. |
| CO₂ Calculation | 43g CO₂ per hard-brake event based on published automotive fuel research (18ml extra fuel × 2.39kg CO₂/litre). Not yet field-validated for BIMSTEC conditions. |
| Impact Numbers | 25% EMS reduction and 18% crash prevention are simulation estimates. Not yet validated by field trials. |

# 5.  Data Stores

## 5.1  In-Memory (Current Submission)

All data lives in Python lists and dicts inside services.py. No database installation needed. Everything works immediately on first run.

## Crash Events  (Python list: _crash_events)

| Field | Name | Type | Description |
| --- | --- | --- | --- |
| ID | event_id | string | Auto-generated e.g. NS-0001 |
| Type | event_type | enum | CRASH ¦ NEAR_MISS ¦ HARD_BRAKE |
| Location | lat / lon | float | GPS coordinates of event |
| Hospital | hospital | string | Nearest hospital (Haversine formula) |
| ETA | ems_eta | float | EMS arrival time in minutes |
| Rider | rider_name, blood_group, allergy | string | Rider medical info |
| Time | timestamp | ISO8601 | UTC time of detection |

## Medical IDs  (Python dict: _medical_db)

| Field | Name | Type | Description |
| --- | --- | --- | --- |
| Name | name | string | Full name of rider |
| Blood | blood_group | string | ABO+Rh e.g. B+ |
| Allergies | allergies | list | Known drug allergies |
| Conditions | conditions | list | Medical conditions |
| Contact | emergency_contact_name/phone | string | Emergency contact details |
| Hospital | nearest_hospital | string | Preferred trauma centre |

## Grey-Spots  (Python dict: GREY_SPOTS)

| Field | Name | Type | Description |
| --- | --- | --- | --- |
| Location | lat / lon | float | Road segment GPS coordinates |
| Count | near | int | Number of near-miss events logged |
| Risk | risk | float | 0.0–1.0 computed risk score |
| Type | type | string | grey ¦ black ¦ blue |
| AI Output | pred | bool | Random Forest blackspot prediction |
| City | city | string | bengaluru ¦ dhaka |

## 5.2  Trained ML Models  (ml/models/)

| File | Model Type | Purpose |
| --- | --- | --- |
| neuroshield_lstm.keras | LSTM (64→32→5) | Full LSTM crash classifier. Requires TensorFlow. |
| neuroshield_lstm.tflite | TFLite quantized | Edge deployment on ESP32. Under 50KB. |
| neuroshield_rf.joblib | RandomForest | Crash classifier fallback. No TensorFlow needed. |
| greyspot_rf.joblib | RandomForest | 6-month blackspot prediction. 100% test accuracy. |
| greyspot_kmeans.joblib | K-Means (k=8) | Near-miss spatial clustering for Bengaluru. |
| greyspot_scaler.joblib | StandardScaler | Feature normalization for grey-spot pipeline. |
| bengaluru_heatmap.geojson | GeoJSON data | Exported heatmap with 12 spatial clusters. |

# 6.  Source Code Map

| File | Location | What it does |
| --- | --- | --- |
| main.py | app/main.py | Flask server. All page routes + API endpoints + SSE /events stream |
| services.py | app/services.py | All business logic — crash, medical, grey-spot, TTS, SSE push |
| base.html | app/templates/base.html | Navigation, dark CSS, SSE connection, shared JavaScript |
| index.html | app/templates/index.html | Overview page — live metrics pulled from API every 3 seconds |
| hospital.html | app/templates/hospital.html | Hospital tab — crash map, SSE feed, V2X panel, alert sound |
| greyspot.html | app/templates/greyspot.html | Grey-spot tab — 2-city Leaflet map, CSV export, spot registry |
| sentinel.html | app/templates/sentinel.html | Sentinel tab — live IMU, LSTM confidence bars, CO₂ counter |
| medical.html | app/templates/medical.html | Medical ID tab — QR scan, create new ID, blood group guide |
| generate_imu_data.py | ml/ | Generates 10,000 synthetic IMU training samples (5 classes) |
| train_lstm.py | ml/ | LSTM training + TFLite export. RandomForest fallback. |
| greyspot_predictor.py | ml/ | K-Means + Random Forest grey-spot ML pipeline |
| edge_emulator.py | ml/ | Interactive Sentinel Node simulator. POSTs to Flask API. |
| run.bat | root | Windows: one-click start (venv + install + run) |
| train_models.bat | root | Windows: train all AI models |
| run_emulator.bat | root | Windows: start Sentinel emulator |